Work and Power Review Problems

(Show work on separate page, and box your clearly labeled answers.)

- A woman pulls a 68.2 kg box across a smooth level floor.  She uses a force of 822 N, directed 27 degrees above horizontal.  The box starts from rest, accelerates at a constant rate, and slides 12.6 meters, at which point it is moving at 4.90 m/s.

  - Make a LARGE free body diagram for the box, and calculate the vector components of the force.

  - Based on her force and the distance, how much work does the woman do on the box?

  - How much kinetic energy does the box have after she pulls it 12.6 meters?

  - What accounts for the difference between her work and the KE of the box?

  - From your free body diagram vectors, calculate the normal force on the box.

  - Calculate the coefficient of friction between the floor and box.

- A small motor is attached to a six strand pulley system which lifts an elevator.  The motor can pulls on a cable with a force of 2800 N, and pulls a distance of 240 meters in a time of 32.0 seconds.

  - What are the work and power output by the motor?

  - How far does the elevator rise?

  - The elevator has a mass of 1445 kg.  What is its weight?

  - How much gravitational potential energy is given to the elevator?

  - What is the efficiency of the motor and pulley system, based on the work input and work output?