Restriction Enzymes and Gel Electrophoresis

Once the DNA is copied by PCR, scientists use restriction enzymes to break the DNA into smaller pieces or fragments.

These fragments can be put into a gel.  Running electricity through the gel causes the fragments to separate out according to size.  Gel electrophoresis gives you the ability to visualize the fragment patterns for comparison.  Scientists look for restriction fragment length polymorphisms (RFLP’s) to solve the case.

Materials:

DNA					UV light box

Gel box with electrical cords		Dye

Power source				Pipette

Agarose gel

Gel tray

Gel comb to make wells

TAE buffer solution

Procedure:

After making the gel, let it cool, put DNA into the wells, and put the gel tray in the gel box.  Add the buffer, hook up the power and let ‘er rip!  The DNA is negatively charged so it will move through the gel towards the positive electrode.  When it is done you can stain the DNA with dye and look at it on a UV light box.  You will be able to see the DNA fragments.

1 piece of DNA is cut into different fragments by

different enzymes

Put DNA in wells.  Each well holds one sample

DNA is pulled to + end.  Smaller fragments move faster

http://en.wikipedia.org/wiki/Agarose_gel_electrophoresis

Can you identify all these things?

http://www.sciencebuddies.org/science-fair-projects/project_ideas/BioChem_p028.shtml

RFLP Analysis – check out these fragments.  Are any of these from the same people?  Is there a match?

http://students.ncwc.edu/bio401/2004/team_b/what_is_gel_electrophoresis.htm

Besides solving crime, what are some other reasons serologists would use these methods?